Smart Irrigation System

Complete Technical Documentation

ESP32 Firmware | MQTT Communication | Node.js Backend | Web Dashboard

April 2026

# Table of Contents

1. Project Overview

2. System Architecture

3. Hardware Components & Wiring

4. ESP32 Firmware

5. MQTT Communication Protocol

6. Backend Server

7. Web Dashboard

8. Installation & Setup Guide

9. API Reference

10. Troubleshooting

# 1. Project Overview

The Smart Irrigation System is an IoT-based solution that automates plant watering based on real-time environmental conditions. It uses an ESP32 microcontroller to read data from three sensors (soil moisture, temperature/humidity, and light), applies intelligent decision logic, and controls a water valve via a relay module. All data is transmitted over WiFi using the MQTT protocol to a central server, which stores historical data and serves a real-time web dashboard for monitoring and manual control.

## Key Features

Fully automatic irrigation based on configurable sensor thresholds

Real-time monitoring via web dashboard with live sensor data and charts

Manual valve override from the web interface

MQTT-based communication for lightweight, real-time data exchange

SQLite database for historical sensor data and event logging

WebSocket-powered live updates (no page refresh needed)

Adjustable thresholds from the dashboard (persisted on ESP32 flash memory)

Safety features: max watering duration, cooldown periods, temperature guards

Docker Compose deployment for easy server setup

# 2. System Architecture

The system follows a three-tier IoT architecture: the edge device (ESP32 with sensors), a message broker (Mosquitto MQTT), and the application layer (Node.js server + web dashboard).

## Architecture Diagram (Text Representation)

+-------------------+         +-------------------+         +-------------------+
|    ESP32 MCU      |  MQTT   |  Mosquitto MQTT   |  MQTT   |  Node.js Server   |
|                   |-------->|     Broker         |-------->|                   |
|  - DHT22 Sensor   |         |  Port: 1883       |         |  - REST API       |
|  - Soil Moisture   |         |  WS: 9001         |         |  - SQLite DB      |
|  - LDR Sensor     |         |                   |         |  - WebSocket      |
|  - Relay/Valve    |<--------|                   |<--------|                   |
+-------------------+  CMDS   +-------------------+  CMDS   +-------------------+
                                                                     |
                                                                     | HTTP/WS
                                                                     v
                                                            +-------------------+
                                                            |  Web Dashboard    |
                                                            |                   |
                                                            |  - Live sensors   |
                                                            |  - Charts         |
                                                            |  - Valve control  |
                                                            |  - Threshold cfg  |
                                                            +-------------------+

## Data Flow

1. The ESP32 reads sensor values every 2 seconds and evaluates irrigation logic locally. 2. Every 5 seconds, it publishes a JSON payload with all sensor readings to the MQTT broker on the topic "irrigation/sensors". 3. The Node.js server subscribes to all irrigation topics and stores readings in SQLite every 30 seconds. 4. The server forwards data in real-time to all connected web dashboard clients via WebSocket. 5. When a user sends a command from the dashboard (e.g., open valve), the server publishes it to "irrigation/valve/command" via MQTT, which the ESP32 receives and acts on.

# 3. Hardware Components & Wiring

## Bill of Materials

| Component | Model/Spec | Quantity | Purpose |
| --- | --- | --- | --- |
| Microcontroller | ESP32 DevKit v1 | 1 | Main controller with WiFi |
| Temp/Humidity Sensor | DHT22 (AM2302) | 1 | Temperature and humidity |
| Light Sensor | LDR + 10K resistor | 1 | Ambient light detection |
| Soil Moisture Sensor | Capacitive v1.2 | 1 | Soil moisture level |
| Relay Module | 5V 1-Channel Relay | 1 | Water valve control |
| Solenoid Valve | 12V Normally Closed | 1 | Water flow control |
| Power Supply | 12V 2A DC Adapter | 1 | Valve + ESP32 power |
| Voltage Regulator | LM7805 or Buck Converter | 1 | 12V to 5V for ESP32 |
| Breadboard + Wires | Standard | 1 set | Prototyping connections |

## Wiring Connections

| Sensor/Module | Pin | ESP32 GPIO | Notes |
| --- | --- | --- | --- |
| DHT22 - Data | OUT | GPIO 4 | 10K pull-up resistor to 3.3V |
| DHT22 - VCC | VCC | 3.3V |  |
| DHT22 - GND | GND | GND |  |
| LDR | Analog Out | GPIO 34 | Voltage divider with 10K resistor |
| Soil Moisture | Analog Out | GPIO 35 | Capacitive sensor preferred |
| Relay Module - Signal | IN | GPIO 26 | Active HIGH |
| Relay Module - VCC | VCC | 5V (Vin) |  |
| Relay Module - GND | GND | GND |  |
| Status LED | + | GPIO 2 | Built-in LED on most ESP32 boards |

### Wiring Notes

The DHT22 sensor requires a 10K ohm pull-up resistor between the data pin and VCC (3.3V). The LDR is connected in a voltage divider configuration with a 10K ohm resistor to GND, and the midpoint is connected to GPIO 34. The capacitive soil moisture sensor is preferred over the resistive type as it does not corrode over time. The relay module controls the solenoid valve; ensure the relay is rated for the valve's current draw. Use a flyback diode across the solenoid coil to protect the relay from voltage spikes.

# 4. ESP32 Firmware

## Overview

The firmware is written in Arduino C++ and is compatible with both Arduino IDE and PlatformIO. It handles all sensor reading, irrigation decision logic, WiFi connectivity, and MQTT communication. Configuration thresholds are stored in non-volatile flash memory using the ESP32 Preferences library, so they survive power cycles.

## File Structure

| File | Description |
| --- | --- |
| smart_irrigation.ino | Main firmware source code |
| platformio.ini | PlatformIO build configuration and library dependencies |

## Required Libraries

| Library | Version | Purpose |
| --- | --- | --- |
| PubSubClient | 2.8+ | MQTT client for Arduino |
| DHT sensor library | 1.4.4+ | Adafruit DHT22 driver |
| Adafruit Unified Sensor | 1.1.9+ | Sensor abstraction layer |
| ArduinoJson | 6.21+ | JSON serialization/deserialization |

## Configuration

Before uploading, edit the following constants at the top of smart_irrigation.ino:

| Constant | Default | Description |
| --- | --- | --- |
| WIFI_SSID | "YOUR_WIFI_SSID" | Your WiFi network name |
| WIFI_PASSWORD | "YOUR_WIFI_PASSWORD" | Your WiFi password |
| MQTT_SERVER | "YOUR_MQTT_BROKER_IP" | IP address of your MQTT broker |
| MQTT_PORT | 1883 | MQTT broker port |
| DEVICE_ID | "esp32_irrigation_01" | Unique device identifier |

## Automatic Irrigation Logic

The firmware evaluates irrigation conditions every 2 seconds using the following decision tree:

1. Check if the system is enabled and not in manual override mode.

2. Check the cooldown period: if the last watering session ended less than 30 minutes ago (configurable), skip.

3. Check temperature safety: if temperature is above 40°C or below 5°C, close the valve and skip.

4. Check light/night conditions: if it is dark and "water at night" is disabled, close the valve and skip.

5. Core decision: if soil moisture is below the minimum threshold (default 30%), open the valve to start watering.

6. Stop condition: if soil moisture reaches the maximum threshold (default 70%), close the valve.

7. Safety timeout: if watering exceeds the maximum duration (default 5 minutes), force-close the valve.

## Default Thresholds

| Parameter | Default Value | Description |
| --- | --- | --- |
| Soil Moisture Min | 30% | Below this level, watering starts |
| Soil Moisture Max | 70% | Above this level, watering stops |
| Temperature Max | 40°C | No watering above this temperature |
| Temperature Min | 5°C | No watering below this temperature |
| Humidity Min | 20% | Low humidity threshold |
| Light Threshold | 50% | Below = nighttime |
| Water at Night | Yes | Whether watering is allowed at night |
| Watering Duration | 5 minutes | Maximum single watering session |
| Cooldown Period | 30 minutes | Minimum gap between sessions |

# 5. MQTT Communication Protocol

MQTT (Message Queuing Telemetry Transport) is a lightweight publish-subscribe messaging protocol ideal for IoT devices with limited bandwidth. The system uses Mosquitto as the MQTT broker, handling all message routing between the ESP32 and the server.

## Topic Structure

| Topic | Direction | Publisher | Description |
| --- | --- | --- | --- |
| irrigation/sensors | ESP32 → Server | ESP32 | Sensor readings (every 5s) |
| irrigation/valve/status | ESP32 → Server | ESP32 | Valve state updates |
| irrigation/valve/command | Server → ESP32 | Server | Valve control commands |
| irrigation/thresholds | Server → ESP32 | Server | Threshold configuration (retained) |
| irrigation/system/status | ESP32 → Server | ESP32 | Device online/offline status |
| irrigation/log | ESP32 → Server | ESP32 | System event logs |

## Sensor Data Payload

Published to irrigation/sensors every 5 seconds:

{
  "device_id": "esp32_irrigation_01",
  "temperature": 25.3,
  "humidity": 62.1,
  "soil_moisture": 45.2,
  "light_level": 78,
  "valve_open": false,
  "manual_override": false,
  "system_enabled": true,
  "uptime_seconds": 3600
}

## Valve Command Payload

Published to irrigation/valve/command:

| Command | JSON Payload | Effect |
| --- | --- | --- |
| Open valve | {"command": "open"} | Manually opens the water valve |
| Close valve | {"command": "close"} | Manually closes the water valve |
| Auto mode | {"command": "auto"} | Returns to automatic control |
| Enable system | {"command": "enable"} | Enables the irrigation system |
| Disable system | {"command": "disable"} | Disables system and closes valve |

# 6. Backend Server

The backend is a Node.js application that acts as the bridge between the MQTT broker and the web dashboard. It subscribes to all MQTT topics, stores data in a SQLite database, exposes a REST API for historical queries, and runs a WebSocket server for real-time updates.

## Technology Stack

| Technology | Purpose |
| --- | --- |
| Node.js | Server runtime |
| Express.js | REST API framework |
| mqtt.js | MQTT client library |
| better-sqlite3 | SQLite database driver (synchronous, fast) |
| ws | WebSocket server for real-time dashboard |
| dotenv | Environment variable management |

## Database Schema

The server uses SQLite with three tables:

sensor_readings: Stores periodic sensor snapshots (every 30 seconds).

| Column | Type | Description |
| --- | --- | --- |
| id | INTEGER PK | Auto-increment ID |
| device_id | TEXT | ESP32 device identifier |
| temperature | REAL | Temperature in °C |
| humidity | REAL | Relative humidity % |
| soil_moisture | REAL | Soil moisture % |
| light_level | INTEGER | Light intensity 0-100% |
| valve_open | INTEGER | 1 = open, 0 = closed |
| timestamp | DATETIME | UTC timestamp |

valve_events: Logs every valve open/close action with source.

system_logs: General system event log (threshold changes, errors, etc.).

# 7. Web Dashboard

The dashboard is a single-page HTML application that connects to the server via WebSocket for real-time sensor updates. It also uses the REST API for historical data and charts.

## Dashboard Features

Live sensor cards: Temperature, Humidity, Soil Moisture, Light Level, and Valve Status with color-coded progress bars.

Valve control panel: Buttons for Open, Close, Auto Mode, and Disable System with visual status indicator.

Historical charts: 24-hour soil moisture trend and temperature/humidity charts powered by Chart.js.

Threshold configuration: Editable form to adjust all irrigation parameters, sent to ESP32 in real-time via MQTT.

System logs: Live scrolling log of all system events.

Responsive design: Works on desktop, tablet, and mobile browsers.

Dark theme: Professional dark interface optimized for readability.

# 8. Installation & Setup Guide

## Prerequisites

ESP32 development board with USB cable

Arduino IDE (2.x) or PlatformIO (VS Code extension)

Node.js 18+ and npm installed on your server/PC

Docker and Docker Compose (recommended for server deployment)

WiFi network accessible by both the ESP32 and server

## Step 1: Flash the ESP32

1. Open the esp32_firmware/ folder in Arduino IDE or PlatformIO.

2. If using Arduino IDE: Install the ESP32 board package via Board Manager. Install all required libraries via Library Manager (PubSubClient, DHT, ArduinoJson).

3. If using PlatformIO: Libraries are defined in platformio.ini and install automatically.

4. Edit the WiFi and MQTT configuration constants at the top of smart_irrigation.ino.

5. Connect the ESP32 via USB and upload the firmware.

6. Open the Serial Monitor at 115200 baud to verify the device boots and connects to WiFi.

## Step 2: Deploy the Server

### Option A: Docker (Recommended)

1. Ensure Docker and Docker Compose are installed on your server.

2. Navigate to the project root directory (where docker-compose.yml is located).

3. Run: docker-compose up -d

4. The MQTT broker will be available on port 1883 and the web dashboard on port 3000.

5. Access the dashboard at http://YOUR_SERVER_IP:3000

### Option B: Manual Setup

1. Install Mosquitto MQTT broker: sudo apt install mosquitto mosquitto-clients

2. Copy mosquitto/mosquitto.conf to /etc/mosquitto/conf.d/ and restart: sudo systemctl restart mosquitto

3. Navigate to the server/ directory and run: npm install

4. Copy .env.example to .env and edit the MQTT broker URL.

5. Start the server: npm start (or npm run dev for development with auto-reload).

6. Access the dashboard at http://localhost:3000

## Step 3: Wire the Hardware

Follow the wiring table in Section 3. Connect all sensors and the relay module to the ESP32 according to the specified GPIO pins. Power the system and verify sensor readings appear in the Serial Monitor and on the web dashboard.

# 9. API Reference

The server exposes a REST API on the configured port (default 3000). All responses are JSON.

| Method | Endpoint | Description |
| --- | --- | --- |
| GET | /api/current | Returns the latest sensor readings and valve status. |
| GET | /api/history?hours=24&limit=1000 | Returns raw historical sensor data for the specified time range. |
| GET | /api/history/hourly?hours=48 | Returns aggregated hourly averages for charting. |
| GET | /api/valve/events?limit=50 | Returns recent valve open/close events. |
| GET | /api/logs?limit=100 | Returns system log entries. |
| GET | /api/stats | Returns 24-hour summary statistics. |
| POST | /api/valve/command | Sends a valve command. Body: {"command": "open¦close¦auto¦enable¦disable"} |
| POST | /api/thresholds | Updates irrigation thresholds. Body: JSON with threshold fields. |

# 10. Troubleshooting

| Issue | Solution |
| --- | --- |
| ESP32 won't connect to WiFi | Verify SSID and password. Ensure 2.4GHz network (ESP32 does not support 5GHz). Check that the router is reachable. Monitor Serial output for error messages. |
| MQTT connection fails | Verify the MQTT broker IP and port. Check that Mosquitto is running (mosquitto status). Ensure no firewall blocks port 1883. Test with: mosquitto_pub -t test -m hello |
| Sensor readings show NaN | Check wiring connections. For DHT22, ensure the pull-up resistor is present. Try a different GPIO pin. Replace the sensor if consistently failing. |
| Soil moisture reads 0% or 100% | Calibrate the sensor: note raw ADC values for dry air (should be ~4095) and water (should be ~0). Adjust the map() range in the firmware accordingly. |
| Valve does not activate | Check relay wiring and power supply. Test the relay independently with a simple HIGH/LOW sketch. Verify the relay trigger voltage (3.3V vs 5V logic). |
| Dashboard shows "Disconnected" | Ensure the Node.js server is running. Check that WebSocket port (same as HTTP) is accessible. Verify the browser console for connection errors. |
| Historical charts are empty | Data is stored every 30 seconds. Wait a few minutes for data to accumulate. Check the database file exists in server/data/irrigation.db. |
| Docker containers won't start | Run docker-compose logs to check errors. Ensure ports 1883 and 3000 are not in use. Verify Docker has sufficient permissions. |

Smart Irrigation System — Technical Documentation — April 2026